[image: image1.jpg]


Pegasus Technologies, Inc.

www.PegasusTech.com   (865) 483-7748

PTSS2003 915 MHz Band Transceiver – Data Sheet

	Features


· FCC Modular Certification ID: QLBPTSS2003 
· XEMICS XE1202 Transceiver

· Customizable TI MSP430 Microcontroller

· RFMD Power Amplifier

· Asynchronous Serial Interface


· Frequency Hopping Spread Spectrum over 160 Channels Ranging From 902 to 928 MHz

· User Selectable Hop Table (65536 Tables)

· Low Power Consumption

· Shut Down mode, < 10uA
· High receiver sensitivity, -107dBm
· SAW filter for high interference rejection.
· 250 mW Transmit Power
· Small Form Factor (1.7” x 1.5” x .35”)

· CRC-16 Error Detection with User Selectable Auto Resend on Error
· User Selectable Data Encryption
· 76,800 bits/sec Data Rate
· 200KHz receive bandwidth
	  Product Overview


The PTSS2003 is a compact, ready-to-use, FCC-certified Frequency Hopping Spread Spectrum transceiver that operates in the 

license-free 902 – 928 MHz band.  It is ideal for OEM applications that require a wireless data connection, and it can also be easily integrated into existing systems.  This versatile module will simplify the task of using wireless to send/receive data, using the latest RFIC technology and a customizable TI MSP430 microcontroller.

	  Applications


· Remote Sensor Monitoring
· Home Automation
· Remote Keyless Entry
· Remote Control
· Security Devices
· Power Sensitive Wireless Applications

[image: image2.png]



The PTSS2003 is only 1.5” x 1.7” x .35”

The PTSS2003 can work in systems with or without an external processor.  When there is no host for the system or for specialized applications, Pegasus can easily customize the microcontroller firmware to meet your needs. 

Low operating power requirements and very low power consumption in shut down mode make this the ideal module for applications where long battery life is important. 

	Electrical Specifications

	
	Min
	Typical
	Max
	Units

	Power Supply Voltage
	2.9 

	3.3
	5.5

	Volts

	Power Supply Current

	Sleep Mode (27 C)
	
	  6
	 20
	uA

	Receive Mode
	
	 30
	 60
	mA

	Transmit Mode
	
	400
	500
	mA

	Serial Host Interface

	‘0’ Voltage Level
	0
	
	 0.6
	V

	‘1’ Voltage Level
	2.4
	
	 3.5
	V

	Host Data Rate
	9600
	
	 115200
	bps

	Analog Channels

	
	
	
	

	Channels




	
	 2
	
	channels

	Input Range
	0
	
	 2.5
	Volts

	Quantization

	
	 12
	
	Bits

	RF Specifications 

	Frequency Range 
	902
	
	 928
	MHz

	Frequency Stability 
	
	 20
	
	ppm

	Architecture



 
	Frequency Hopping Spread Spectrum

	Channels per hop table
	50

	
	
	channels

	Available channels

	
	 48000

	
	channels

	Packet Length
	 8
	
	760
	bits

	Receiver Sensitivity
	
	-107
	
	dBm

	Out of Band Rejection
	
	 40

	
	dB

	Transmitter

	RF Output Power

	
	250

	
	 mW

	Modulation


	
	2-FSK
	
	

	Data Rate
	
	76800
	
	bps

	Deviation

	
	(100
	
	KHz


	Environmental Conditions

	Operating Temperature
	-40 C to + 80 C

	Relative Humidity
	5% to 95%  Non Condensing


	Mechanical Dimensions

	Length
	1.70 ± 0.01
	inch

	Width
	1.50 ± 0.01
	inch

	Height
	0.35 ± 0.02
	inch











Specifications are subject to change without notice. This device complies with Part 15 of the FCC rules.  Operation is subject to the following conditions. 1) This device may not cause harmful interference, and 2) This device must accept any interference received, including interference that may cause undesired operation.


